I’ll show it as a simple visual flow: people → local AI → Teamcenter/ERPNext → answer, with the Internet completely outside the factory network.

For your factory, I would deploy it in **five physical/network layers** so that the LLM is isolated, auditable, and never needs Internet access.

## 1. The big picture

Think of your factory network like a secure building with several locked rooms:

```text

FACTORY NETWORK

┌───────────────────────────────────────────────────────────────┐

│                                                               │

│  Room 1             Room 2             Room 3                 │

│  USERS              AI SERVICES        DATA SYSTEMS           │

│                                                               │

│ Engineers   --->   AI Gateway   --->   Teamcenter             │

│ Planners           Agent Server       ERPNext                 │

│ Managers           LLM Server         Document DB             │

│                    GPU Servers                                │

│                                                               │

│                         │                                     │

│                         v                                     │

│                  Logging / Monitoring                         │

│                                                               │

└───────────────────────────────────────────────────────────────┘

X

X FIREWALL

X

NO INTERNET

```

The important idea is:

```text

User computers

↓

AI application

↓

Local LLM

↓

Controlled tools

↓

Teamcenter / ERPNext

```

The LLM should **never directly connect to the databases**.

---

# 2. Physical servers I would deploy

For a real production environment, I would not put everything on one computer.

I would start with approximately **6–8 servers**.

```text

Server 1   AI Application Server

Server 2   AI Application Server / standby

Server 3   GPU LLM Server

Server 4   GPU LLM Server / standby or scale-out

Server 5   Vector / AI Database Server

Server 6   Logging + Monitoring Server

Server 7   Internal Container Registry

Server 8   Backup Server

```

Your existing:

```text

Teamcenter servers

ERPNext servers

Database servers

```

remain separate.

---

# 3. GPU server — the most important computer

This is the machine actually running the LLM.

A good configuration today would look roughly like:

```text

┌─────────────────────────────────────┐

│       AI GPU SERVER                 │

│                                     │

│ CPU                                 │

│ 2 × AMD EPYC / Intel Xeon           │

│                                     │

│ RAM                                 │

│ 512 GB – 1 TB ECC                   │

│                                     │

│ GPU                                 │

│ 2–4 × NVIDIA GPUs                   │

│                                     │

│ Local storage                       │

│ 2 × 1 TB NVMe OS                    │

│ 4–8 TB NVMe model storage           │

│                                     │

│ Network                             │

│ 25 / 100 GbE                        │

│                                     │

│ OS                                  │

│ RHEL / Rocky Linux / Ubuntu Server  │

└─────────────────────────────────────┘

```

For example, NVIDIA's current RTX PRO 6000 Blackwell Server Edition has **96 GB ECC GPU memory per GPU**. ([NVIDIA][1])

So:

```text

1 GPU  =  96 GB VRAM

2 GPUs = 192 GB VRAM

4 GPUs = 384 GB VRAM

```

That is important because LLM size is largely constrained by GPU memory.

---

# 4. What I would buy for your first deployment

I would probably start with:

```text

2 × GPU servers

Each:

---------------------------------

2 × CPUs

512 GB RAM minimum

2 × RTX PRO 6000 96GB GPUs

2 × 1TB NVMe boot

2 × 4TB enterprise NVMe

2 × 25GbE NIC

Dual redundant PSU

TPM

Secure Boot

BMC management

```

This gives each server:

```text

192 GB GPU memory

```

which is enough for fairly substantial local models and gives you room for quantization, long context, and concurrent users.

NVIDIA's current certified-system guidance recommends system RAM at least roughly **2× total GPU memory**, balanced PCIe topology, NVMe local storage, TPM 2.0, and appropriate high-speed networking for multi-node inference. ([NVIDIA Docs][2])

---

# 5. Why two GPU servers?

Suppose you use only:

```text

GPU Server A

```

and it fails.

Then:

```text

Factory AI = DOWN

```

Instead:

```text

Load Balancer

|

+------+------+

|             |

v             v

GPU Server A   GPU Server B

```

If A fails:

```text

Load Balancer

|

v

GPU Server B

```

Your AI system continues running.

---

# 6. AI application servers

Do **not** let users directly call the LLM server.

Instead:

```text

User

↓

AI Application

↓

LLM

```

The application server handles:

```text

Authentication

Authorization

User sessions

Prompt construction

Tool selection

Audit logging

Teamcenter access

ERPNext access

LLM access

```

Typical hardware can be much smaller:

```text

AI Application Server

CPU:    16–32 cores

RAM:    64–128 GB

Disk:   1–2 TB NVMe

NIC:    10/25 GbE

GPU:    not required

```

Software:

```text

Linux

Docker / Podman

FastAPI

Python

Agent framework

Nginx / HAProxy

```

---

# 7. Separate the AI application from the LLM

This distinction is very important.

```text

AI Application Server

|

|

Internal API

|

v

LLM Server

```

For example:

```text

AI App

|

| POST /v1/chat/completions

|

v

10.30.30.21:8000

|

Local LLM

```

The user should never know or access:

```text

10.30.30.21

```

directly.

---

# 8. Network segmentation

This is probably more important than the exact LLM you choose.

I would create separate VLANs/security zones.

```text

FACTORY NETWORK

┌────────────────────┐

│ VLAN 10            │

│ USER NETWORK       │

│                    │

│ Engineering PCs    │

│ Management PCs     │

└─────────┬──────────┘

|

FIREWALL

|

┌─────────v──────────┐

│ VLAN 20            │

│ AI APPLICATION     │

│                    │

│ Web UI             │

│ API Gateway        │

│ Agent Servers      │

└─────────┬──────────┘

|

FIREWALL

|

+--------+--------+

|                 |

v                 v

┌─────────────────┐  ┌──────────────────┐

│ VLAN 30         │  │ VLAN 40          │

│ AI GPU NETWORK  │  │ BUSINESS DATA    │

│                 │  │                  │

│ GPU server A    │  │ Teamcenter       │

│ GPU server B    │  │ ERPNext          │

│ Local LLM       │  │ DB / PLM         │

└─────────────────┘  └──────────────────┘

```

---

# 9. Firewall rules

This is where you control exactly who can talk to whom.

For example:

### User → AI application

Allow:

```text

VLAN 10 → VLAN 20

TCP 443

HTTPS

```

Nothing else.

---

### AI application → LLM

Allow:

```text

VLAN 20 → VLAN 30

TCP 8000

```

or your internal HTTPS endpoint.

---

### AI application → Teamcenter

Only permit Teamcenter's required service/API ports.

```text

VLAN 20

|

| approved Teamcenter APIs

v

VLAN 40

```

---

### AI application → ERPNext

For example:

```text

VLAN 20

|

| HTTPS 443

v

ERPNext REST API

```

Do not allow:

```text

LLM server → ERPNext DB

```

or:

```text

LLM server → Teamcenter DB

```

---

# 10. What the LLM network can access

Ideally:

```text

GPU VLAN

|

+---- AI application server

|

+---- internal monitoring

|

+---- internal registry

```

Nothing else.

Especially:

```text

GPU VLAN

X

X Internet

X

```

There should literally be **no default route to the Internet**.

---

# 11. Internet isolation

There are two levels.

### Level A — firewall isolation

```text

Factory

|

Firewall

|

Internet

```

with outbound traffic blocked.

This is usually called a disconnected environment.

### Level B — true air gap

```text

Factory network

physical separation

Internet network

```

There is no physical network route whatsoever.

For your stated security level, I would consider **true air gap** if company policy supports it.

---

# 12. How do you get models inside?

This is an important question because your LLM server cannot access Hugging Face or NVIDIA.

You use a separate **staging machine**.

```text

INTERNET SIDE

Approved Download Workstation

|

|

v

Download model

Download container

Download dependencies

|

v

Security scan

|

v

Sign / checksum

|

v

Approved transfer

```

Then:

```text

USB / controlled transfer system

|

v

Security inspection

|

v

Air-gap import station

|

v

Internal registry

```

NVIDIA's current NIM documentation specifically supports this two-phase workflow: prepare the model/container assets on a connected machine and transfer the staged files into the isolated environment; the air-gapped NIM then runs from local storage without Internet or API credentials. ([NVIDIA Docs][3])

---

# 13. Internal container registry

You don't want production machines attempting:

```text

docker pull docker.io/...

```

Instead run your own:

```text

registry.factory.local

```

For example:

```text

registry.factory.local/

ai/

llm-nim:3.0

agent-service:1.4

embedding-service:2.1

reranker:1.0

```

Possible internal registry products:

```text

Harbor

Red Hat Quay

JFrog Artifactory

Sonatype

```

Then:

```text

Internet staging

|

approved transfer

|

v

Internal Registry

|

+--> GPU servers

+--> App servers

+--> Kubernetes

```

Air-gapped Kubernetes deployments similarly require local copies of containers and deployment artifacts such as Helm charts. ([NVIDIA Docs][4])

---

# 14. Model storage

Models can consume hundreds of GB.

So create a dedicated storage area:

```text

/model-store/

qwen/

model-v1/

model-v2/

llama/

model-v1/

embeddings/

model-v1/

```

For example:

```text

GPU Server

|

+--- NVMe local model cache

|

+--- Central secure storage

```

I'd use enterprise NVMe because model startup can involve reading very large weight files.

---

# 15. RAG/document server

You probably have:

```text

engineering procedures

SOPs

manufacturing manuals

work instructions

quality documents

machine manuals

Teamcenter metadata

```

These can be indexed locally.

```text

Documents

|

v

Embedding model

|

v

Vector database

```

Example:

```text

PostgreSQL

+

pgvector

```

This server does **not** need a big GPU.

Typical:

```text

CPU: 16–32 cores

RAM: 128–256 GB

NVMe: 4–10 TB

```

depending on document volume.

---

# 16. Keep the embedding model local too

Sometimes people deploy:

```text

Local LLM

```

but accidentally call:

```text

OpenAI embedding API

```

That breaks the security model.

Instead:

```text

Factory documents

|

v

Local embedding model

|

v

pgvector

```

Everything remains inside.

---

# 17. Teamcenter integration

I would create a Teamcenter connector service:

```text

AI Agent

|

v

Teamcenter Connector

|

v

Teamcenter APIs

```

Example tools:

```text

get_item()

get_revision()

get_ebom()

get_release_status()

get_change_notice()

get_dataset_metadata()

```

The LLM doesn't need to understand Teamcenter's internal DB structure.

---

# 18. ERPNext integration

Same approach:

```text

AI Agent

|

v

ERPNext Connector

|

REST API

|

ERPNext

```

Functions such as:

```text

get_item()

get_inventory()

get_bom()

get_work_order()

get_purchase_order()

get_stock_balance()

```

---

# 19. Why not database access?

Avoid:

```text

LLM

|

v

SQL Server

```

because the LLM could accidentally produce:

```sql

DELETE ...

UPDATE ...

DROP ...

```

or simply query data a particular user should not see.

Instead:

```text

LLM

|

v

Agent

|

v

Approved function

|

v

ERPNext / Teamcenter

```

---

# 20. Identity should propagate

Suppose engineer John asks:

```text

"Show the BOM for Project X."

```

The system should know:

```text

John

↓

AI Application

↓

John's permissions

↓

Teamcenter

```

not:

```text

Everyone

↓

AI

↓

Super Administrator account

```

That is very important.

Ideally integrate with:

```text

Active Directory

LDAP

Kerberos

OIDC

```

Then the architecture becomes:

```text

Active Directory

|

v

User → AI application

|

v

Authorization

|

+------+------+

|             |

v             v

Teamcenter      ERPNext

```

---

# 21. Logging server

Every AI action should be auditable.

Example:

```text

11:02:15

User: john.smith

Question:

"Can we produce 500 AX-100?"

Tool:

Teamcenter.get_bom("AX-100")

Tool:

ERPNext.get_inventory(...)

Result:

420 units

Model:

Qwen-X version 2.4

```

Store these logs separately.

```text

AI Application

|

+------> Audit Server

GPU Server

|

+------> Monitoring Server

```

Possible stack:

```text

Prometheus

Grafana

OpenSearch

ELK

Splunk

```

all local.

---

# 22. Management network

Servers should also have a completely separate management network.

```text

Production network

|

AI Servers

|

─────────────

|

Management NIC

|

v

VLAN 99

```

Only administrators can access VLAN 99.

For example:

```text

BMC

iDRAC

iLO

IPMI

Redfish

switch management

```

should never be visible from user VLANs.

---

# 23. Example VLAN design

A realistic layout might look like:

```text

VLAN 10

Engineering users

10.10.10.0/24

VLAN 20

AI applications

10.10.20.0/24

VLAN 30

GPU inference

10.10.30.0/24

VLAN 40

Teamcenter

10.10.40.0/24

VLAN 50

ERPNext

10.10.50.0/24

VLAN 60

AI databases / RAG

10.10.60.0/24

VLAN 70

Monitoring

10.10.70.0/24

VLAN 80

Internal registry

10.10.80.0/24

VLAN 90

Backup

10.10.90.0/24

VLAN 99

Hardware management

10.10.99.0/24

```

The exact numbers don't matter; the **separation** does.

---

# 24. Switch infrastructure

For normal client/server traffic:

```text

10 GbE

```

is more than enough.

For GPU servers and heavy storage:

```text

25 GbE / 100 GbE

```

is preferable.

For multi-node inference or very large models:

```text

100 / 200 / 400 GbE

```

can become relevant. NVIDIA's certified-system guidance calls for at least **200 Gbps for multi-node inference**, depending on the architecture. ([NVIDIA Docs][2])

Your first system probably does **not** need 400 Gb networking.

I would likely use:

```text

User network       1/10 Gb

Application        10/25 Gb

Database           10/25 Gb

GPU servers        25/100 Gb

Storage            25/100 Gb

Management         1 Gb

```

---

# 25. Physical switch layout

A sensible layout:

```text

Core Switch A

||

||

Core Switch B

||

+---------++----------+

|                     |

v                     v

AI Switch A             AI Switch B

|                       |

+----+----+              +---+----+

|         |              |        |

GPU-1    App-1          GPU-2    App-2

```

Each important server has:

```text

NIC 1 → Switch A

NIC 2 → Switch B

```

So one switch can fail without taking down your AI system.

---

# 26. Storage architecture

You have two choices.

### Simpler

Keep models locally:

```text

GPU server

|

NVMe

|

model files

```

This gives excellent performance.

### More enterprise

```text

Shared storage

/              \

/                \

GPU Server A      GPU Server B

```

using:

```text

NAS

SAN

NVMe-oF

```

I would still maintain local caches.

---

# 27. Backup system

Backup:

```text

Agent configuration

Vector DB

Prompts/templates

Audit logs

Container images

Model metadata

Certificates

Configuration

```

You don't necessarily need to back up model weights every night because they are immutable and can be restored from your approved package.

Architecture:

```text

Production

|

v

Backup server

|

v

Offline backup

```

For very sensitive environments, consider immutable/offline backups.

---

# 28. High availability architecture

Ultimately:

```text

Internal DNS

|

v

Load Balancer

/            \

/              \

App-1              App-2

|                  |

+--------+---------+

|

AI Gateway

|

+---------+---------+

|                   |

v                   v

GPU-1               GPU-2

|                   |

+---------+---------+

|

Local Model Store

+--------------+--------------+

|                             |

v                             v

Teamcenter                    ERPNext

```

No single server failure should stop the system.

---

# 29. Docker vs Kubernetes

For your first proof of concept:

```text

Docker / Podman

```

is perfectly reasonable.

For production with multiple GPU nodes:

```text

Kubernetes

```

can be useful.

NVIDIA currently supports NIM on Docker, Kubernetes, Helm, NIM Operator, KServe, multi-node environments, and air-gapped deployments. ([NVIDIA Docs][5])

I would **not start with Kubernetes just because it is fashionable**.

Start:

```text

2 GPU servers

Docker / NIM or vLLM

```

and move to Kubernetes when you need:

```text

multiple models

automatic failover

GPU scheduling

many applications

rolling updates

multiple teams

```

---

# 30. Deployment process

I'd deploy in this order.

### Phase 1 — isolated proof of concept

```text

1 GPU server

1 application server

1 vector DB

```

Only test data.

No Teamcenter writes.

No ERPNext writes.

---

### Phase 2 — read-only factory integration

Connect:

```text

Teamcenter read APIs

ERPNext read APIs

```

Users can ask:

```text

What is the latest revision?

How many parts are available?

Which work orders are delayed?

Does ERPNext BOM match Teamcenter?

```

Still:

```text

NO WRITE

```

---

### Phase 3 — production security

Add:

```text

AD authentication

RBAC

audit logs

firewall segmentation

HA

backup

model approval process

```

---

### Phase 4 — controlled actions

Eventually permit things such as:

```text

create report

create draft work order

create change request

```

but require confirmation:

```text

AI:

"I am about to create Work Order WO-1234.

Proceed?"

↓

User approves

↓

ERPNext API executes

```

---

# 31. My recommended first production hardware

For your environment, I'd start approximately here:

```text

QUANTITY

AI Application server      2

GPU inference server       2

Vector DB / RAG server     2

(primary + standby)

Logging server             1–2

Internal registry          1–2

Backup server              1

Management server          1

```

GPU server:

```text

2 × enterprise CPUs

512 GB – 1 TB ECC RAM

2 × 96 GB GPU

8–16 TB NVMe

2 × 25/100 GbE

redundant PSU

TPM 2.0

Secure Boot

BMC

```

Application servers:

```text

16–32 CPU cores

128 GB RAM

2 TB NVMe

2 × 10/25 GbE

```

Database/RAG:

```text

32 CPU cores

256 GB RAM

4–10 TB enterprise NVMe

2 × 10/25 GbE

```

---

## The most important architecture

If you remember only one diagram, make it this:

```text

┌───────────────┐

│ Factory Users │

└───────┬───────┘

│ HTTPS

▼

┌────────────────┐

│ AI Application │

│ + Security     │

│ + Permissions  │

│ + Audit        │

└───────┬────────┘

│

┌───────────┼──────────────┐

│           │              │

▼           ▼              ▼

┌────────┐  ┌──────────┐  ┌──────────┐

│Local   │  │Teamcenter│  │ ERPNext  │

│LLM/GPU │  │ + NX     │  │          │

└────────┘  └──────────┘  └──────────┘

│

▼

Local RAG

Documents

══════════════════════════════════════════

FACTORY SECURITY BOUNDARY

══════════════════════════════════════════

❌

│

NO ROUTE

│

🌐

Internet

```

The strongest security property here is that **the GPU/LLM is not the trusted security authority**. Your application layer, firewall, identity system, and Teamcenter/ERPNext permissions remain responsible for access control. The LLM is just a reasoning engine inside that protected environment.

If you want, I can next turn this into a **rack-by-rack diagram** showing exactly where the GPU servers, switches, firewalls, Teamcenter, ERPNext, storage, and backup servers physically connect.

[1]: https://www.nvidia.com/en-us/data-center/rtx-pro-6000-blackwell-server-edition/?utm_source=chatgpt.com "RTX PRO 6000 Blackwell Server Edition | NVIDIA"

[2]: https://docs.nvidia.com/certification-programs/latest/nvidia-certified-configuration-guide.html?utm_source=chatgpt.com "NVIDIA-Certified Systems Configuration Guide — NVIDIA Certification Programs Documentation"

[3]: https://docs.nvidia.com/nim/large-language-models/3.0.0/deployment/air-gap-deployment.html?utm_source=chatgpt.com "Air-Gap Deployment — NVIDIA NIM for Large Language Models"

[4]: https://docs.nvidia.com/datacenter/dps/versions/latest/guides/installation/airgap-deployment/?utm_source=chatgpt.com "Dynamic Power Software Docs"

[5]: https://docs.nvidia.com/nim/large-language-models/1.15.0/deployment-guide.html?utm_source=chatgpt.com "Deployment Guide for NVIDIA NIM for LLMs — NVIDIA NIM for Large Language Models (LLMs)"